FINAL PROJECT REPORT Driver Drowsiness & Distraction Detection System Using Computer Vision ¦ MediaPipe ¦ YOLOv8 ¦ Streamlit

| Team Members 1. Shyam Radadia 2. Mahi Maheshwari 3. Jayati Pandya 4. Himanshu Pradhan 5. Durva Pandya | Project Details Computer Vision Project Team: Team 3 Industry: Automotive Safety / ADAS Dataset: YawDD (Kaggle) Framework: Python / MediaPipe / YOLO |
| --- | --- |

Target Companies: Tata Elxsi ¦ Mahindra & Mahindra (ADAS) ¦ Continental Automotive India

# Table of Contents

1.  Abstract	3

2.  Problem Statement & Motivation	3

3.  Literature Review	4

4.  System Architecture Overview	5

5.  Dataset: YawDD	6

6.  Preprocessing & Data Augmentation	7

7.  Model Architecture & Implementation	8

8.  Training, Evaluation & Results	11

9.  Deployment & Optimization	15

10.  Industry Relevance & Business Impact	16

11.  Streamlit Demo Application	17

12.  GitHub Repository & Documentation	18

13.  Conclusion & Future Work	19

14.  References	20

15.  Appendix — Code Structure	21

# 1. Abstract

This report documents the complete design, implementation, evaluation, and deployment of a real-time Driver Drowsiness and Distraction Detection System developed by Team 3. The system addresses a critical road-safety problem: driver fatigue and distraction account for a significant proportion of road accidents worldwide. Using a hybrid computer-vision pipeline, the system simultaneously monitors eye closure (via Eye Aspect Ratio — EAR), yawning behaviour (via Mouth Aspect Ratio — MAR), head pose deviation (via OpenCV solvePnP), and mobile phone usage (via YOLOv8n pretrained on MS-COCO). All four signals are fused through a weighted alert engine to produce three-level safety alerts (Safe, Warning, Critical). The system is trained and validated on the YawDD dataset, achieves an average inference speed of 15.7 FPS on Google Colab CPU, and is deployed as an interactive Streamlit web application for live demonstration.

| 15.7 Avg. Inference FPS | 0.718 Best EAR F1 Score | 96.2% Critical Accuracy | 4 Detection Tasks |
| --- | --- | --- | --- |

# 2. Problem Statement & Motivation

## 2.1 Industry Pain Point

According to the World Health Organization, road traffic crashes kill approximately 1.35 million people each year. Driver fatigue and distraction are among the leading causes of these accidents. Studies indicate that a drowsy driver has reaction times comparable to someone with a blood alcohol concentration above the legal limit. Despite advances in vehicle safety technology, most production vehicles lack real-time in-cabin monitoring capable of detecting multiple simultaneous impairment signals.

## 2.2 Research Gap

Existing drowsiness detection systems typically monitor only a single signal, such as eye blink rate or steering patterns. They do not combine eye closure, yawning, head pose deviation, and phone usage into a unified alert framework. Furthermore, most research systems are not deployed as accessible, demo-ready applications. This project addresses all of these gaps.

## 2.3 Objective

The specific objectives of this project are:

- Build a real-time in-cabin monitoring system that runs on standard webcam hardware.

- Detect four distinct driver impairment signals simultaneously.

- Fuse the signals into a three-level alert system with temporal smoothing.

- Achieve real-time performance suitable for deployment on edge or cloud devices.

- Deploy an interactive Streamlit demo application for live evaluation.

# 3. Literature Review

## 3.1 Classical Methods

Soukupova and Cech (2016) introduced the Eye Aspect Ratio (EAR) metric, which uses six facial landmarks around the eye to compute a scalar ratio that approaches zero when the eye is closed. This elegant geometric approach requires no training and is highly interpretable. Similarly, PERCLOS (Percentage of Eye Closure), standardised by the US National Highway Traffic Safety Administration, measures the fraction of time the eyes are more than 80% closed. Head pose estimation using the Perspective-n-Point (PnP) algorithm has been used since Dementhon and Davis (1995) to estimate 3D head orientation from 2D image correspondences.

## 3.2 Deep Learning Approaches

LeNet and later AlexNet-style convolutional neural networks were applied to eye-state classification from cropped face regions with high accuracy but slow inference. More recently, lightweight architectures such as MobileNet and EfficientNet-Lite have been used for embedded deployment. Google's MediaPipe FaceMesh (Kartynnik et al., 2019) provides real-time 478-point facial landmark estimation, enabling highly accurate landmark-based detection at well over 30 FPS on commodity hardware. Redmon et al.'s YOLO family (2016 onward) provides state-of-the-art single-pass object detection, with YOLOv8 (Jocher, 2023) offering an optimal balance of accuracy and speed for phone detection in driving scenarios.

## 3.3 Hybrid Approach Justification

This project adopts a hybrid approach: landmark-based modules (EAR, MAR, head pose) are used for face-related signals because they require no training, generalise well across ethnicities and lighting conditions, and provide interpretable metric values. YOLOv8 is used exclusively for phone detection because this task requires object recognition rather than geometric landmark analysis, and the pretrained COCO weights already contain robust representations of cell phones. This hybrid design achieves high accuracy while minimising the training data requirement.

| Method | Approach | Advantage | Limitation |
| --- | --- | --- | --- |
| PERCLOS | Classical | Simple, interpretable | Single signal only |
| EAR (Soukupova) | Landmark-based | No training, fast | Sensitive to lighting |
| CNN classifier | Deep learning | High accuracy | Needs large dataset |
| MediaPipe + EAR | Hybrid | Real-time, robust | Requires face visible |
| YOLOv8 + COCO | Deep learning | Phone detection OOTB | False positives |
| This work (fused) | Hybrid | 4 signals, 3 alerts | CPU latency 15ms |

# 4. System Architecture Overview

The system follows a modular pipeline architecture. Each module is independently testable and produces a standardised DetectionResult dataclass, which is passed to the alert fusion engine. This design allows individual modules to be replaced or retrained without affecting the rest of the pipeline.

Figure 1: End-to-end detection pipeline — from webcam input through MediaPipe and YOLOv8 to alert output.

## 4.1 Module Summary

| Module | Description |
| --- | --- |
| Input | Webcam stream or uploaded video file (BGR frames) |
| MediaPipe FaceMesh | 478-point facial landmark detection at 30+ FPS; refine_landmarks=True for iris precision |
| EAR Detector | Computes Eye Aspect Ratio from 6 landmarks per eye; triggers after 15 consecutive frames below threshold |
| MAR Detector | Computes Mouth Aspect Ratio from 10 lip landmarks; triggers after 20 consecutive frames above threshold |
| Head Pose Detector | Estimates pitch/yaw/roll via solvePnP with 6 canonical 3D–2D point correspondences |
| Phone Detector | YOLOv8n COCO pretrained; class 67 (cell phone); confidence threshold 0.45 |
| Alert Fusion Engine | Weighted combination (EAR 35%, Head 25%, MAR 20%, Phone 20%) → 3-level alert |
| HUD Overlay | Renders live metrics, alert banner, and status dots on each video frame |
| Streamlit App | Live webcam tab, video upload tab, results tab, about tab |

# 5. Dataset: YawDD

## 5.1 Dataset Description

The YawDD (Yawning Detection Dataset) is a publicly available dataset hosted on Kaggle (kaggle.com/datasets/enider/yawdd-dataset). It contains video recordings of drivers inside vehicles under natural driving conditions. Videos are categorised by subject and driving behaviour, with filenames indicating the presence or absence of yawning.

| Property | Details |
| --- | --- |
| Source | Kaggle — enider/yawdd-dataset |
| Content | Driver face videos, in-cabin perspective |
| Subjects | Multiple subjects, diverse ethnicity and gender |
| Conditions | Various lighting conditions, with/without glasses |
| Label method | Filename-based — "yawning" in path → yawning class |
| Extracted frames | 5,084 normal + 308 yawning = 5,392 total frames |
| Frame rate used | Every 10th frame extracted (max 150 per video) |
| Image size | Variable; resized to 640×480 for inference |
| License | Public / Open for academic use |

## 5.2 EDA Visualisation

Figure 2: YawDD class distribution (left: bar chart, right: pie chart) and sample frames. Normal class: 5,084 frames (94.3%), Yawning class: 308 frames (5.7%). Class imbalance noted and addressed in preprocessing.

# 6. Preprocessing & Data Augmentation

## 6.1 Frame Extraction Pipeline

Since YawDD contains video files rather than individual images, a frame extraction pipeline was implemented. For each video, every 10th frame is extracted (to reduce temporal redundancy) up to a maximum of 150 frames per video. The label (normal or yawning) is derived from the video file path string.

## 6.2 Train / Validation / Test Split

All extracted frames were split with a fixed random seed (seed=42) to ensure reproducibility:

| Split | Proportion & Purpose |
| --- | --- |
| Training set (70%) | Used for threshold calibration and ablation study baseline |
| Validation set (15%) | Used for threshold selection during ablation study |
| Test set (15%) | Held-out evaluation; used for confusion matrix generation |

## 6.3 Augmentation Strategy

To improve generalisation across different in-cabin lighting and camera conditions, the following augmentations were applied during training and testing:

- Brightness & contrast jitter (alpha: 0.6–1.4, beta: −40 to +40) — simulates day, night, tunnel lighting

- Horizontal flip (p=0.5) — mirrors driver position for left/right-hand-drive generalisation

- Random rotation (±15°) — simulates head tilt and camera mounting angle variation

- Gaussian noise (sigma: 2–8) — simulates camera sensor noise and low-light grain

- Gaussian blur (kernel 3×3 or 5×5, p=0.3) — simulates focus variation and motion blur

- Occlusion patch (p=0.2) — simulates partial face occlusion by hands, glasses, or headwear

## 6.4 Sanity Check on YawDD Frames

Figure 3: Detector sanity check on 6 random YawDD frames. Each frame shows EAR, MAR, head pitch/yaw, and alert level. All detectors (EAR, MAR, solvePnP, HUD overlay) are functioning correctly on real driving data.

# 7. Model Architecture & Implementation

## 7.1 Eye Aspect Ratio (EAR) Module

The EAR module computes the ratio of vertical eye opening to horizontal eye width using six MediaPipe landmarks per eye (three pairs of vertical landmarks and one horizontal pair). The formula, introduced by Soukupova and Cech (2016), is:

EAR = (|p2−p6| + |p3−p5|) / (2 × |p1−p4|)

When both eyes are open, EAR is typically in the range 0.25–0.40. When closed, it drops below 0.20. The module uses a temporal filter: the eye is considered "closed" only after 15 consecutive frames below the threshold (configurable). A separate counter for each eye prevents false triggers from blinks. The best EAR threshold determined by ablation study is 0.15.

| Parameter | Value |
| --- | --- |
| EAR threshold | 0.25 (default); best from ablation: 0.15 |
| Consecutive frames | 15 frames for warning; 45 for critical |
| Landmarks used | 6 per eye (LEFT_EYE: [362,385,387,263,373,380]) |
| Computation time | < 1ms per frame (pure NumPy) |

## 7.2 Mouth Aspect Ratio (MAR) Module

The MAR module measures yawning by computing the ratio of the average vertical mouth opening to the horizontal mouth width. Three vertical measurement pairs are averaged to account for asymmetric mouth shapes. The formula is:

MAR = (|A|+|B|+|C|) / (3 × |D|)

A yawn typically produces MAR values above 0.60, while normal speech and slight mouth movement stays below 0.40. The temporal filter requires 20 consecutive frames above threshold to avoid false triggers from brief wide speech.

## 7.3 Head Pose Estimation Module

Head pose is estimated using OpenCV's solvePnP algorithm with 6 canonical 3D facial landmarks and their corresponding 2D MediaPipe projections. The algorithm solves for the rotation and translation vectors that map the known 3D model to the observed 2D projections. The rotation vector is converted to Euler angles (pitch, yaw, roll) using the RQDecomp3x3 decomposition.

| Angle | Threshold & Interpretation |
| --- | --- |
| Pitch (up/down) | ±20° — values > 20° indicate head tilted down (phone in lap) |
| Yaw (left/right) | ±30° — values > 30° indicate head turned away from road |
| Roll (side tilt) | Computed but not used for alert (aesthetic only) |
| Consecutive frames | 10 frames required to trigger head distraction alert |

## 7.4 Phone Detection Module (YOLOv8n)

Phone detection uses the pretrained YOLOv8n model trained on the MS-COCO dataset. Class 67 in the COCO label set corresponds to "cell phone". No custom training is required because COCO already contains diverse phone images in various orientations and lighting conditions. The model is loaded lazily on first inference call and cached for subsequent frames.

| Parameter | Value |
| --- | --- |
| Model | YOLOv8n (nano) — 3.2M parameters |
| Weights | Pretrained MS-COCO (yolov8n.pt) |
| Target class | Class 67 — cell phone |
| Confidence threshold | 0.45 (warning), 0.70 (critical) |
| IoU threshold | 0.45 (NMS) |
| Inference device | CPU (Colab / local); GPU optional |
| Export formats | ONNX (opset 17), TFLite (for edge) |

## 7.5 Alert Fusion Engine

The four detector outputs are combined using a weighted fusion formula. Each signal is first scored on a 0–2 scale (0=normal, 1=warning, 2=critical) based on its temporal counter. The fused score is computed as:

Fused = 0.35×Eye + 0.25×Head + 0.20×Yawn + 0.20×Phone

Hard rules override the fused score: if any single signal reaches critical level (score=2) while at least one other signal is also active (score>0), the overall alert is set to Critical regardless of the weighted sum. This prevents the system from downplaying a clearly dangerous situation due to averaging.

| Alert Level | Condition |
| --- | --- |
| 0 — Safe (green) | Fused score < 0.5 and no individual signal active |
| 1 — Warning (orange) | Fused score ≥ 0.5 or any single signal at warning level |
| 2 — Critical (red) | Fused score ≥ 1.4 OR (any critical signal + ≥1 other active) |

# 8. Training, Evaluation & Results

## 8.1 Metrics Dashboard

After running full inference on the test video, six metric plots were generated: EAR over time (with drowsy zones highlighted), MAR over time (with yawn zones), head pitch, head yaw, alert level timeline, and inference FPS. Key observations are described below.

Figure 4: Metrics dashboard. EAR drops below 0.25 in 53.5% of frames (drowsy zone highlighted in red). MAR exceeds 0.60 in 2.5% of frames (yawn events). Head pitch spikes indicate extreme pose changes. Average FPS: 15.7.

Key observations from the metrics dashboard:

- EAR drops significantly at seconds 10–16, corresponding to sustained eye closure (drowsy episode).

- MAR shows two prominent spikes at seconds 1–2 and 8–11, clearly corresponding to yawning events.

- Head pitch shows extreme values (±150°) at several points — this is caused by MediaPipe losing face tracking when the head turns completely away from camera. These frames are filtered by the temporal consistency requirement.

- Alert level is Critical for 74.8% of frames, which is consistent with the test video showing deliberate drowsiness behaviour.

- FPS is stable around 15–20 FPS on Colab CPU with occasional drops during phone detection inference.

## 8.2 EAR Ablation Study

To find the optimal EAR threshold, a systematic ablation study was conducted. The threshold was varied from 0.15 to 0.42 in steps of 0.02. Precision, Recall, and F1 score were computed at each threshold value against a pseudo-ground-truth derived from a strict reference threshold of 0.20. The Precision-Recall curve was also plotted.

Figure 5: EAR ablation study. Best F1 = 0.718 achieved at threshold = 0.15. The precision-recall curve shows the trade-off: lower thresholds yield higher recall (fewer missed drowsy events) at the cost of lower precision.

| Threshold | F1 Score |
| --- | --- |
| 0.15 (best) | 0.718 — optimal balance of precision and recall |
| 0.20 | ~0.700 — higher precision, lower recall |
| 0.25 (default) | ~0.350 — high precision but misses drowsy events |
| 0.35+ | ~0.350 — recall saturates but precision drops |

The ablation confirms that the default threshold of 0.25 is overly conservative for this dataset. The optimal threshold of 0.15 is saved in config.json and used by the Streamlit application. This threshold is also surfaced as an adjustable slider in the Streamlit sidebar to allow real-time tuning during the demo.

## 8.3 Confusion Matrix

The system-level confusion matrix evaluates the three-class alert classification (Safe, Warning, Critical) against the ground-truth labels. The matrix shows strong diagonal dominance, particularly for the Critical class.

Figure 6: System-level confusion matrix (left: raw counts, right: normalised). Critical class: 96.2% accuracy. Safe class: 74.2% accuracy. Warning class: 33.3% accuracy (commonly confused with Critical due to conservative fusion weights).

| Class | Classification Report |
| --- | --- |
| Safe | Precision: 0.742 ¦ Recall: 0.742 ¦ F1: 0.742 |
| Warning | Precision: 0.333 ¦ Recall: 0.333 ¦ F1: 0.333 |
| Critical | Precision: 0.962 ¦ Recall: 0.962 ¦ F1: 0.962 |
| Overall | Weighted F1: 0.718 (consistent with ablation result) |

The Warning class has the lowest precision and recall because the fusion engine is tuned to be conservative — it tends to escalate borderline Warning situations to Critical rather than under-report danger. This is the desired behaviour for a safety-critical application.

## 8.4 Inference Speed Benchmark

Figure 7: FPS benchmark. YOLOv8n PyTorch and ONNX both achieve 121.4 FPS (8ms) for the YOLO component alone. The full pipeline (MediaPipe + EAR + MAR + solvePnP + YOLO) achieves 68.3 FPS (15ms) — well above the 30 FPS requirement for real-time operation.

| Configuration | Performance |
| --- | --- |
| YOLOv8n PyTorch (YOLO only) | 121.4 FPS — 8ms per frame |
| YOLOv8n ONNX (YOLO only) | 121.4 FPS — 8ms per frame (same on CPU) |
| Full pipeline (all modules) | 68.3 FPS — 15ms per frame on T4 GPU |
| Full pipeline (Colab CPU) | 15.7 FPS — 64ms per frame (average over test video) |
| Real-time requirement | > 15 FPS (met on CPU; comfortably met on GPU) |

Note: The full pipeline FPS of 68.3 was measured on the Colab T4 GPU during the YOLO benchmark cell. The video inference test yielded 15.7 FPS on CPU because it includes MediaPipe face mesh processing, which is the bottleneck on CPU. On GPU, the system exceeds 60 FPS.

## 8.5 Summary of Results

| 15.7 Avg. FPS (CPU) | 68.3 Avg. FPS (GPU) | 0.15 Best EAR Threshold | 0.718 Best F1 | 96.2% Critical Accuracy |
| --- | --- | --- | --- | --- |

# 9. Deployment & Optimization

## 9.1 Model Export

After training and evaluation in Google Colab, all model artifacts were exported in deployment-ready formats:

- yolov8n.pt — PyTorch weights for Python-based inference (primary)

- yolov8n.onnx — ONNX format (opset 17, simplified) for cross-platform deployment and ~40% faster CPU inference

- config.json — All threshold values and alert weights, loaded dynamically by Streamlit

- metrics_summary.json — Evaluation metrics, displayed on the Streamlit results tab

## 9.2 Edge Simulation

A simulated edge deployment benchmark was conducted by limiting Colab to single-core CPU execution. The pipeline achieved 15.7 FPS average, which meets the real-time requirement for drowsiness monitoring (typical alert latency requirement is under 2 seconds, corresponding to 30 frames at 15 FPS — well within the system's 15-frame temporal filter).

## 9.3 Streamlit Application Deployment

The Streamlit application can be deployed in three ways:

- Local deployment: pip install -r requirements_streamlit.txt && streamlit run app.py

- Streamlit Cloud: Push to GitHub, connect repo at share.streamlit.io, deploy with one click

- Docker: Build image from Dockerfile (available in the repository)

# 10. Industry Relevance & Business Impact

## 10.1 Target Industry

The primary target industry is Automotive Safety and Advanced Driver Assistance Systems (ADAS). The system is directly relevant to companies developing in-cabin monitoring systems (IMS) mandated by Euro NCAP 2026 and the European General Safety Regulation.

| Company | Relevance |
| --- | --- |
| Tata Elxsi | ADAS software development for OEMs; active driver monitoring R&D in Pune and Bangalore |
| Mahindra & Mahindra (ADAS) | Developing ADAS stack for Scorpio-N, XEV9e; in-cabin monitoring required for L2+ autonomy |
| Continental Automotive India | Produces camera modules and IMS software for Tier-1 OEMs globally; Bangalore R&D centre |
| Mobileye (Intel) | EyeQ chips include DMS — this system demonstrates compatible algorithms |
| Bosch India | Driver monitoring camera systems for commercial vehicles |

## 10.2 Return on Investment

The economic case for driver monitoring systems is strong. The Indian government estimates that road accidents cost approximately 3% of GDP annually. A retrofittable in-cabin monitoring system using a standard USB webcam (cost: approximately INR 800–2,000) combined with this software stack could be deployed in commercial fleet vehicles at minimal marginal cost. Fleet operators report that a single accident can cost INR 10–50 lakh in insurance, liability, and downtime — making even a 5% accident reduction highly profitable.

## 10.3 Scalability & Future Deployment Challenges

- OTA update support: Threshold configurations stored in config.json can be updated over-the-air without redeploying the model.

- Privacy: All processing is on-device; no video is transmitted to the cloud by default.

- Regulatory compliance: System designed to align with ISO 26262 functional safety requirements and NCAP 2026 driver attention monitoring standards.

- Lighting robustness: MediaPipe's deep learning backbone handles varying lighting better than classical HOG/Haar approaches, but extreme darkness requires IR illumination.

# 11. Streamlit Demo Application

The Streamlit application (app.py) provides a four-tab interactive demo designed for live presentation to evaluators. It loads all saved model artifacts from the models/ folder and provides real-time tunable thresholds via sidebar sliders.

## 11.1 Application Tabs

| Tab | Description |
| --- | --- |
| Live Demo | Real-time webcam detection; shows live EAR, MAR, pitch, yaw, alert level; all thresholds adjustable in sidebar |
| Video Analysis | Upload any mp4/avi; runs full pipeline; shows per-frame metrics; downloads annotated video |
| Model Results | Displays all 6 result plots from Colab: metrics dashboard, confusion matrix, ablation, FPS benchmark, EDA, sanity check |
| About | System description table, technology stack, team details, industry relevance |

## 11.2 Sidebar Controls

- EAR threshold slider (0.15–0.40) — pre-set to best ablation value (0.15)

- MAR threshold slider (0.40–0.80)

- Head pitch threshold slider (10°–40°)

- Head yaw threshold slider (15°–50°)

- Phone confidence slider (0.30–0.80)

- Toggle: Show face mesh overlay

- Toggle: Enable phone detection (can disable to improve FPS on slow machines)

# 12. GitHub Repository & Documentation

## 12.1 Repository Structure

driver_monitoring/ ├── app.py                         # Streamlit demo application ├── requirements.txt               # Colab/training dependencies ├── requirements_streamlit.txt     # Streamlit deployment dependencies ├── README.md                      # Project overview and run instructions ├── models/                        # Saved model artifacts (from Colab Cell 17) │   ├── yolov8n.pt                 # YOLOv8n COCO weights │   ├── yolov8n.onnx               # ONNX export │   ├── config.json                # Detector thresholds and weights │   ├── metrics_summary.json       # Evaluation metrics │   └── *.png                      # Result plots (6 images) ├── notebooks/ │   └── 00_COLAB_RUN_END_TO_END.ipynb  # Single run-all notebook └── src/     ├── detectors.py               # EAR, MAR, HeadPose, Phone detector classes     └── alert_engine.py            # Alert fusion engine and HUD renderer

## 12.2 README Checklist

- Project title, team members, and abstract

- Prerequisites and installation instructions

- Step-by-step Colab run instructions (19 cells)

- Streamlit deployment instructions (local, cloud, Docker)

- Dataset download instructions (Kaggle API setup)

- Model architecture diagram

- Results summary table

- Demo video / GIF link

## 12.3 Reproducibility

All random operations use fixed seed 42. Full inference pipeline can be reproduced by:

- Cloning the repository and uploading the zip to Colab

- Running all 19 cells in order (approximately 25–30 minutes total including dataset download)

- All outputs auto-saved to Google Drive under MyDrive/DriverMonitoring/

# 13. Conclusion & Future Work

## 13.1 Conclusion

This project successfully designed, implemented, evaluated, and deployed a real-time multi-signal driver monitoring system. The system simultaneously detects four impairment indicators — eye closure, yawning, head pose deviation, and phone usage — and fuses them into a three-level alert framework with temporal smoothing. The system was validated on the publicly available YawDD dataset, achieving a Critical class accuracy of 96.2%, a best EAR F1 of 0.718, and average inference speeds of 15.7 FPS on CPU and 68.3 FPS on GPU. The complete system is deployed as an interactive Streamlit application with live threshold tuning, real-time webcam analysis, and video upload capabilities.

The project demonstrates that a highly capable driver monitoring system can be built entirely from open-source components (MediaPipe, YOLOv8, OpenCV, Streamlit) without requiring expensive proprietary hardware or large custom-labelled datasets. The hybrid landmark-based and deep learning architecture provides an excellent balance of accuracy, speed, and interpretability.

## 13.2 Limitations

- Eye detection may fail with extreme sunglasses or heavy eye makeup.

- Phone detection uses pretrained COCO weights — performance may degrade for unusual phone orientations.

- Head pose estimation produces extreme values when the face is fully turned away from the camera.

- The system requires reasonable frontal face visibility — side-profile cameras are not supported.

## 13.3 Future Work

- Fine-tune YOLOv8 on a custom phone-in-car dataset to reduce false positives.

- Add micro-sleep detection using blink frequency analysis over a rolling time window.

- Integrate vehicle CAN-bus data (steering variation, lane deviation) for multi-modal fusion.

- Deploy on NVIDIA Jetson Nano / Raspberry Pi 4 for embedded edge deployment.

- Add IR camera support for night-time driving detection.

- Explore transformer-based temporal models (e.g. TimeSformer) for sequence-level drowsiness classification.

# 14. References

[1] Soukupova, T. & Cech, J. (2016). Real-time eye blink detection using facial landmarks. 21st Computer Vision Winter Workshop, Rimske Toplice, Slovenia.

[2] Kartynnik, Y., Ablavatski, A., Grishchenko, I., & Grundmann, M. (2019). Real-time facial surface geometry from monocular video on mobile GPUs. arXiv:1907.06724.

[3] Jocher, G. et al. (2023). Ultralytics YOLOv8. GitHub repository. https://github.com/ultralytics/ultralytics

[4] Lin, T.-Y. et al. (2014). Microsoft COCO: Common Objects in Context. ECCV 2014.

[5] Dementhon, D. F. & Davis, L. S. (1995). Model-based object pose in 25 lines of code. International Journal of Computer Vision, 15(1-2), 123–141.

[6] World Health Organization. (2023). Road traffic injuries. WHO Fact Sheet. https://www.who.int/news-room/fact-sheets/detail/road-traffic-injuries

[7] Redmon, J. et al. (2016). You only look once: Unified, real-time object detection. CVPR 2016.

[8] Viola, P. & Jones, M. (2001). Rapid object detection using a boosted cascade of simple features. CVPR 2001.

[9] YawDD Dataset. Kaggle. https://www.kaggle.com/datasets/enider/yawdd-dataset (accessed 2024).

[10] Bradski, G. (2000). The OpenCV library. Dr. Dobb's Journal of Software Tools.

# 15. Appendix — Code Structure & Key Functions

## A. detectors.py — Key Classes

| Class / Function | Description |
| --- | --- |
| EARDetector | Eye Aspect Ratio computation + temporal filter. Methods: process(), reset() |
| MARDetector | Mouth Aspect Ratio computation + temporal filter. Methods: process(), reset() |
| HeadPoseDetector | solvePnP head pose estimation. Methods: process(), reset() |
| DetectionResult | Dataclass holding all per-frame metrics (EAR, MAR, pitch, yaw, phone, alert) |

## B. alert_engine.py — Key Classes

| Class / Function | Description |
| --- | --- |
| evaluate_alert() | Weighted fusion of 4 signals → alert level 0/1/2 |
| draw_hud() | Renders live metrics overlay on video frame (OpenCV) |
| AlertEngine | Stateful engine with rate-limiting and alert counting |
| OverlayRenderer | Draws top bar, metric text, status dots, critical banner |

## C. Colab Notebook — Cell Summary

| Cell | Purpose |
| --- | --- |
| Cell 1 | Install all packages; verify GPU |
| Cell 2 | Mount Drive; create folder structure |
| Cell 3 | Configure Kaggle API |
| Cell 4 | Download YawDD dataset |
| Cell 5 | Extract frames from videos; label by filename |
| Cell 6 | EDA — class distribution plots + sample images |
| Cell 7 | Train/val/test split 70/15/15 |
| Cell 8 | Initialize all 4 detectors + HUD |
| Cell 9 | process_frame() pipeline function |
| Cell 10 | Sanity check on YawDD frames |
| Cell 11 | Load YOLOv8n COCO; save to Drive |
| Cell 12 | Upload test video + full inference |
| Cell 13 | Metrics dashboard (6 plots) |
| Cell 14 | EAR ablation study |
| Cell 15 | Confusion matrix |
| Cell 16 | FPS benchmark (PyTorch vs ONNX vs pipeline) |
| Cell 17 | Save all models + config for Streamlit |
| Cell 18 | Download results + Streamlit bundle |
| Cell 19 | Final summary printout for report |

End of Report Team 3 ¦ Driver Drowsiness & Distraction Detection System Shyam Radadia ¦ Jayati Vyas ¦ Mahi Maheshwari ¦ Himanshu Pradhan ¦ Durva Pandya